GUIDANCE NOTE 02

# Engineering Certification — What We Look For

Usually one element missing, rather than the whole document.

Most engineering we receive is fine. When we do come back on it, it is almost always a single missing element rather than a missing document — and almost always one of the items below.

## What We Need to See

- The engineer's name, company and registration number

- Signed and dated

- The wind region and terrain category the design is based on

- Site classification where the design depends on it

- The specific structure and address, or clearly matched to them

- Footing details — depth, diameter, and reinforcement

- Tie-down and bracing details

- Any conditions or limits the engineer has set

## Common Reasons We Come Back

- Generic manufacturer's engineering not matched to the wind region for the site

- Undated, or dated before the design it certifies

- Certification covering a different span, height or roof load than the plans show

- A condition set in the engineering that the drawings don't reflect

- A page missing from a multi-page set

## What an Acceptable Certification Says

- "Designed in accordance with AS 1170.2 for Wind Region A, Terrain Category 2."

- "Footings: 450 mm diameter × 900 mm deep, N12 bar as detailed."

- "Maximum span 6.0 m. Not suitable for spans exceeding this without further advice."

- "This certification applies to the structure shown on drawing 24-0198 rev C, dated 14 May 2026."

- Signed, dated, with the engineer's name and registration number below the signature.

Lodge the same engineering often? Save it once under My Documents in the portal and it's a tick at lodgement from then on — no re-upload; replace it there when the engineer revises it, and every later job carries the new set.

If the engineering sets a condition, make sure the plans show it. A minimum footing depth or a maximum span in the engineering has to appear on the drawings too — we check one against the other.