GUIDANCE NOTE — DRAFT

# Pool & spa barriers — what the plans must show

Barriers are assessed to AS 1926.1. These are the items that decide it.

DRAFT — NOT PUBLISHED. Figures below are the commonly applied AS 1926.1 requirements — the registered building surveyor to verify each against the current standard and WA regulations before release.

## The numbers that get checked every time

| Barrier height | Minimum 1200 mm, measured on the outside (approach side), anywhere along the barrier. |
| --- | --- |
| Gap under the barrier | No more than 100 mm from finished ground. |
| Non-climbable zone | A 900 mm quadrant on the outside of the barrier clear of footholds — no horizontal rails, retaining, planters, BBQs or trees inside it. |
| Gates | Self-closing and self-latching from any position, opening away from the pool, latch release at least 1500 mm above ground (or internally shielded). |
| Boundary fence as barrier | Effective height on the inside of at least 1800 mm with its own non-climbable zone on the inside face. [Confirm quoted figure.] |
| Windows & doors | Doors are not acceptable as part of the barrier for new pools; openable windows into the pool area are restricted. [Confirm current WA position wording.] |

## What the site plan must show

- The barrier line drawn right around the pool zone — fences, walls, gates, and which existing fences are being relied on.

- Gate positions and swing direction.

- Heights of every barrier element, including existing boundary fences being relied on.

- Anything inside the non-climbable zones — retaining, equipment, landscaping — so we can see it's clear.

- The pool/spa itself dimensioned to boundaries, with the engineering or manufacturer's specification attached.

The common bounce: a compliant fence with a climbable object beside it — a retaining wall, a raised planter, the filter box. Walk the barrier line on the plan and ask "could a five-year-old use this?" That's the assessment.

## After the permit

Local governments inspect pool barriers on an ongoing cycle after construction — keeping the barrier compliant is a permanent obligation, not a one-off. [Insert current WA inspection interval.]