GUIDANCE NOTE 05

# Elevations — What Yours Need to Show

Every dimension we have to check, on the drawing itself.

Elevations are where height, clearance and pitch get checked, and where a missing figure stops an assessment. The two below are typical — an attached patio and a freestanding shed — with the dimensions we look for marked on them.

[ diagram: 1Overall height, measured from natural ground level 2Clearance to the underside of the beam, or wall height 3Roof pitch or fall, stated as a figure 4Overall width and post or column spacing, dimensioned 5Natural ground line, and finished floor or slab level 6How the structure attaches to the existing dwelling ]

## What We Need to See

- Every elevation that shows the works — usually all four for a freestanding structure

- Overall height measured from natural ground level, not the slab

- Wall height, or clearance to the underside of the beam

- Roof pitch or fall stated as a figure, not just drawn

- Post and column positions, dimensioned

- Natural ground line, plus finished floor or slab level

- The fixing detail where it attaches to the dwelling, or a reference to it

- Materials and finishes

- Scale, or a note that the drawing is not to scale

## Common Reasons We Come Back

- Only one elevation supplied for a freestanding structure

- Overall height taken from the slab rather than natural ground

- Roof pitch drawn but never stated as a figure

- Natural ground line left off on a sloping site

- Post spacing not dimensioned, so it can't be checked against the engineering

- Elevations that don't match the floor plan they came with

Heights are measured from natural ground level, not the slab. On a sloping site, show the ground line across the full elevation. It is regularly the difference between a job going straight through and one coming back.