GUIDANCE NOTE 07

# Lodging Checklist

Everything we need, by job type — check it before you send.

Nearly every job that comes back to us is missing one item from this page. None of it is unusual — it is simply what has to be in front of us before an assessment can be finished.

## Every Job, Whatever the Work

### Site Plan

- Every lot boundary dimensioned

- The proposed structure dimensioned and located

- Setback to every boundary — not just the close ones

- All existing structures on the lot

- North point and street frontage

- Easements, sewer and drainage where they affect the works

- Scale, or a note that it isn't to scale

### Elevations

- Every elevation showing the works — usually all four if freestanding

- Overall height from natural ground level, not the slab

- Wall height, or clearance under the beam

- Roof pitch or fall stated as a figure

- Post and column positions dimensioned

- Natural ground line and finished floor level

### Engineering

- Signed and dated

- Engineer's name, company and registration number

- Wind region and terrain category

- Footing details — depth, diameter, reinforcement

- Tie-down and bracing

- Matched to this structure and address

- Any conditions the engineer has set, shown on the drawings too

A plan with too much on it has never delayed a job. A plan missing one dimension routinely does.

## Then, Depending on the Work

### Patio, Pergola or Verandah

- How it attaches to the dwelling — the fixing detail or a reference to it

- Gutter and downpipe positions, and where stormwater discharges

- Roof sheet profile and colour

- Where it sits within 900 mm of a boundary, the fire separation detail

### Shed, Garage or Workshop

- Manufacturer's engineering matched to the wind region

- Slab detail, or the footing design if no slab

- Door and opening sizes

- Wall and roof cladding, and colour

- If it's to be habitable in any way, say so — it changes the class

### Carport

- Open on the required sides — show which

- Setback to the street boundary

- Fixing detail where attached to the dwelling

### Retaining Wall

- Structural engineering design for the wall

- Existing and proposed levels on both sides

- Retained height measured from the top of the footing

- Surcharge where a driveway, structure or fill sits above

- Drainage behind the wall, and where it discharges

- Setback from the boundary, and how a boundary wall is dealt with

## Then, Depending on the Work — Continued

### Bushfire Prone Lot

- The BAL assessment itself, not the rating quoted in an email

- Prepared by an accredited person, dated, for this lot

- Where BAL-40 or BAL-FZ, construction details on the plans to match

### Swimming Pool or Spa

- Barrier location and type shown on the site plan

- Gate positions and swing direction

- Climbable zones on both sides of the barrier

- Where a boundary fence forms part of the barrier, its height and construction

## Before You Press Send

| Does everything match? | Site plan, elevations and engineering should describe the same structure. Mismatched spans, heights or post spacings between drawings and engineering are the most common reason we come back. |
| --- | --- |
| Is the engineering current? | Dated before the design it certifies, or covering a different span or height, and we can't use it. |
| Are conditions reflected? | A minimum footing depth or maximum span set by the engineer has to appear on the drawings as well. We check one against the other. |
| Is it legible? | PDFs, right way up, text selectable where possible. A photograph of a drawing on a desk is rarely readable enough to assess. |

Not sure whether something applies? Ask before you lodge rather than after. It takes a moment at our end and saves you a request for further information — and the days that go with it.