GUIDANCE NOTE 09

# Swimming Pool & Spa Barriers

Barriers are assessed to AS 1926.1. These are the items that decide it, and what your plans must show.

## The Numbers That Get Checked Every Time

| Barrier height | Minimum 1200 mm, measured on the outside (approach side), anywhere along the barrier. |
| --- | --- |
| Gap under the barrier | No more than 100 mm from finished ground. |
| Non-climbable zone | A 900 mm quadrant on the outside of the barrier clear of footholds — no horizontal rails, retaining, planters, BBQs or trees inside it. |
| Gates | Self-closing and self-latching from any position, opening away from the pool, latch release at least 1500 mm above ground (or internally shielded). |
| Boundary fence as barrier | Effective height on the inside of at least 1800 mm, with the non-climbable zone on the inside face. |
| Windows & doors | Doors can't form part of the barrier for a new pool; openable windows into the pool area are restricted (limited opening or screened). |

## What the Site Plan Must Show

- The barrier line drawn right around the pool zone — fences, walls, gates, and which existing fences are being relied on.

- Gate positions and swing direction.

- Heights of every barrier element, including existing boundary fences being relied on.

- Anything inside the non-climbable zones — retaining, equipment, landscaping — so we can see it's clear.

- The pool or spa itself dimensioned to boundaries, with the engineering or manufacturer's specification attached.

The common bounce: a compliant fence with a climbable object beside it — a retaining wall, a raised planter, the filter box. Walk the barrier line on the plan and ask "could a five-year-old use this?" That's the assessment.

## After the Permit

Pool barriers stay a permanent obligation, not a one-off: your local government inspects them on an ongoing cycle (at least once every four years in WA). Keep the barrier compliant as landscaping and fences change.