GUIDANCE NOTE 10

# Wind Class & Site Classification

The two letters on the engineering that have to match your actual site.

## Wind Class (AS 4055): N1–N6

Every structure is engineered for a wind class, and the class belongs to the site, not the product. Around Perth (non-cyclonic Region A) most suburban lots land at N1–N2, but the same shed on a coastal, open-rural or hilltop lot can jump to N3 or higher. What moves it:

- Terrain — open water or paddocks versus built-up suburbia.

- Shielding — neighbouring buildings that break the wind, or the lack of them.

- Topography — crests, escarpments and slopes accelerate wind.

Generic "N2" brochure engineering on an exposed N3 site is the classic mismatch — and the first thing we check.

## Site Classification (AS 2870): A, S, M, H, E, P

The soil class drives footing and slab design — from A/S (sand, stable — most of the Perth coastal plain) through M/H/E (increasingly reactive clays, common in the hills and river soils) to P (problem sites: fill, variable ground). Slabs and footings must be designed for the class that exists, not assumed sand.

## What We Look for at Assessment

- The engineering states a wind class, and that class is plausible for the site's terrain, shielding and topography.

- Footing and slab details match the soil class — with a soil report where the ground is reactive, filled, or the engineer requires one.

- Site plan features that change the answer are visible: coastal frontage, open rural outlook, crest of a hill, cut and fill.

Not sure what your site is? Say so in the lodgement notes rather than guessing — confirming a wind or soil class early is quick; re-engineering after assessment is not. Our engineering-on-demand service can also close the gap.