GUIDANCE NOTE 11

# Stormwater & Soak Wells

Every new roof sheds water somewhere. The plan has to show where — and it can't be next door.

## The Rule, in One Line

Perth councils expect stormwater to be contained on your own lot. On the sandy soils of the coastal plain that almost always means soak wells; on clay soils in the hills it takes designed storage or a lawful connection instead — soak wells barely work in clay. Roof water discharging over a boundary is both a neighbour dispute and a non-compliance, and it's the first thing checked when a new roof area is added.

## What We Need to See

- Soak well locations marked on the site plan, with sizes (diameter × depth, or litres) for the new roof area.

- Downpipe positions and which soak well each connects to.

- Clearance from buildings, footings and boundaries — as a rule of thumb about 1.8 m unless the design says otherwise. A soak well hard against a footing undermines it; hard against a boundary, it drains the neighbour's ground too.

- Overflow path — where water goes in the storm the wells can't hold, falling away from buildings and not over the boundary.

- On clay or filled ground: the designed alternative — storage, connection to a council system where permitted, or an engineer's drainage detail.

## Common Reasons We Come Back

- No stormwater shown at all — the single most common gap on Class 10 site plans.

- A soak well drawn under the new shed or its footings, with no design for it.

- Downpipes on the plan that don't connect to anything.

- New paving or patio area draining to the street or the neighbour.

- Soak wells on a hills block with reactive clay — where they won't actually soak.

Sizing: councils vary on the design storm they require, and some publish their own soak-well sizing tables — check your council's page (linked under Resources → Your local government) or ask us and we'll tell you what your council expects.