GUIDANCE NOTE 12

# Easements, Sewer & Drainage

The lines on the title you can't see on site — and who has to agree before you build over them.

## Three Things That Run Through Ordinary Lots

- Easements on the title — drainage, access, power. An easement usually means no structures without the beneficiary's consent, and it binds you even if nothing is visible on the ground.

- Water Corporation sewer mains — often through the rear or side of the lot, not the street. Building over or near one needs Water Corporation approval before the permit stage, and the answer shapes footing design.

- Council drainage — pipes and pits in dedicated easements, with the same no-build presumption.

## Before the Design Is Settled

- Pull the certificate of title — every easement is listed on it. We can help if you don't have a copy.

- Check the service locations — a Before You Dig (BYDA) search is free and shows sewer, water, power and gas in and around the lot.

- If the structure lands on or near a sewer: talk to Water Corporation first. Their approval (and any concrete-encasement or pier requirements) has to exist before the council will issue the permit.

## What We Need to See

- Easements and any sewer within the lot drawn on the site plan — or a note that the title is clear.

- The structure dimensioned clear of them, or the written approval that lets it encroach, with footings designed to match.

- Soak wells kept clear of sewer lines and easements too — they count.

The expensive version of this note: a slab poured over an unprotected sewer junction. It gets found — usually when the neighbour's line blocks — and rectification means cutting the slab. A five-minute title and BYDA check at design time is the whole prevention.