QUICK REFERENCE — PIN THIS UP

# Lodging a complete job

A complete lodgement is the single biggest thing that speeds a job up. Guidance note 07 in the portal has the detail; this card is the checklist.

## Every job, every time

- Site plan — the whole lot, the new structure dimensioned to boundaries, north point, and existing buildings shown. (The #1 reason jobs bounce — see note 01.)

- Elevations — every relevant face, with heights and boundary relationships dimensioned. (Note 05.)

- Engineering — signed and dated structural certification matching the drawings, with the site wind class stated. (Note 02.)

- PDFs only, up to 40 MB per lodgement — bigger sets, email the office first.

- Pick the structures on the form — the description writes itself; saved engineering can be ticked from My Documents. Repeating a past job? Lodge Similar carries it across.

## Then add, by job type

### Patio · carport (Class 10a)

✓ Connection details to the dwelling (if attached) ✓ Fire separation shown if within 900 mm of a boundary ✓ BAL rating evidence if bushfire prone — the form asks for it before lodgement

### Shed · garage (Class 10a)

✓ Slab/footing detail ✓ Wind class on the engineering matching the site ✓ Boundary setbacks dimensioned (or on-boundary noted) ✓ New BAL report if bushfire prone and the house is 2016+ (older: exempt)

### Pool · spa + barrier (Class 10b)

✓ Barrier line marked on the site plan ✓ Gate positions and swing direction ✓ Non-climbable zones achievable — fence heights shown ✓ Pool + barrier engineering/specification

### Retaining wall (Class 10b)

✓ Levels both sides of the wall ✓ Drainage behind the wall shown ✓ Engineering for the retained height ✓ Distance to boundaries and structures (Note 04.)

## Before you press lodge

- Is the address exactly right — including lot number and suburb?

- Do the drawings, engineering and address all refer to the same job and revision?

- Bushfire prone? The form checks the map. A pre-2016 house is exempt; from 2016 a shed needs its own new report, a patio/carport the rating evidence — the form won't lodge without it. Reports come from an accredited bushfire assessor, not us.

- Strata? The form asks Yes or No — a Yes makes the strata plan a required upload.

- Does the design meet R-Codes setbacks, or do you have planning approval? A compliant certificate still stalls at the council if planning isn't settled.

Not sure something's needed? Lodge with a note in the "Notes for CFBA" box — it goes straight to the assessing surveyor — or message us on any job. Asking first is always faster than a bounced application.